2. Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

CODE:

clc;

clear;

close all;

% Input Specifications

Fs = 1000;       % Sampling frequency (Hz)

Fp = 100;        % Passband edge (Hz)

Fst = 150;       % Stopband edge (Hz)

Rp = 1;          % Passband ripple (dB)

Rs = 40;         % Stopband attenuation (dB)

% Normalized Frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

% Chebyshev Type-I Low-Pass Filter

[Nc_LP, Wn_LP] = cheb1ord(Wp, Ws, Rp, Rs);

[bC_LP, aC_LP] = cheby1(Nc_LP, Rp, Wn_LP, 'low');

% Chebyshev Type-I High-Pass Filter

% Swap passband and stopband edges for HPF

[Nc_HP, Wn_HP] = cheb1ord(Wp, Ws, Rp, Rs);

[bC_HP, aC_HP] = cheby1(Nc_HP, Rp, Wn_HP, 'high');

% Butterworth Low-Pass Filter

[Nb_LP, Wb_LP] = buttord(Wp, Ws, Rp, Rs);

[bB_LP, aB_LP] = butter(Nb_LP, Wb_LP, 'low');

% Butterworth High-Pass Filter

[Nb_HP, Wb_HP] = buttord(Wp, Ws, Rp, Rs);

[bB_HP, aB_HP] = butter(Nb_HP, Wb_HP, 'high');

% Display Filter Orders and Coefficients

disp('CHEBYSHEV TYPE-I LOW-PASS FILTER');

disp(['Filter Order = ', num2str(Nc_LP)]);

disp('Numerator Coefficients:');

disp(bC_LP);

disp('Denominator Coefficients:');

disp(aC_LP);

disp('CHEBYSHEV TYPE-I HIGH-PASS FILTER');

disp(['Filter Order = ', num2str(Nc_HP)]);

disp('Numerator Coefficients:');

disp(bC_HP);

disp('Denominator Coefficients:');

disp(aC_HP);

disp('BUTTERWORTH LOW-PASS FILTER ORDER');

disp(Nb_LP);

disp('BUTTERWORTH HIGH-PASS FILTER ORDER');

disp(Nb_HP);

% Frequency Responses

[HC_LP, f] = freqz(bC_LP, aC_LP, 2048, Fs);

[HC_HP, ~] = freqz(bC_HP, aC_HP, 2048, Fs);

[HB_LP, ~] = freqz(bB_LP, aB_LP, 2048, Fs);

[HB_HP, ~] = freqz(bB_HP, aB_HP, 2048, Fs);

% Low-Pass Comparison

figure;

plot(f, 20*log10(max(abs(HC_LP),1e-10)), 'b');

hold on;

plot(f, 20*log10(max(abs(HB_LP),1e-10)), 'r--');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Low-Pass Filter Comparison');

legend('Chebyshev Type-I','Butterworth');

xline(Fp, '--k', 'Passband Edge');

xline(Fst, ':k', 'Stopband Edge');

ylim([-100 5]);

% High-Pass Comparison

figure;

plot(f, 20*log10(max(abs(HC_HP),1e-10)), 'b');

hold on;

plot(f, 20*log10(max(abs(HB_HP),1e-10)), 'r--');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('High-Pass Filter Comparison');

legend('Chebyshev Type-I','Butterworth');

xline(Fp, '--k', 'Passband Edge');

xline(Fst, ':k', 'Stopband Edge');

ylim([-100 5]);

% Chebyshev Low-Pass Magnitude and Phase

figure;

subplot(2,1,1);

plot(f, 20*log10(max(abs(HC_LP),1e-10)));

grid on;

title('Chebyshev Type-I Low-Pass Magnitude Response');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

subplot(2,1,2);

plot(f, unwrap(angle(HC_LP))*180/pi);

grid on;

title('Chebyshev Type-I Low-Pass Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

% Chebyshev High-Pass Magnitude and Phase

figure;

subplot(2,1,1);

plot(f, 20*log10(max(abs(HC_HP),1e-10)));

grid on;

title('Chebyshev Type-I High-Pass Magnitude Response');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

subplot(2,1,2);

plot(f, unwrap(angle(HC_HP))*180/pi);

grid on;

title('Chebyshev Type-I High-Pass Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

% Pole-Zero Plots

figure;

zplane(bC_LP, aC_LP);

title('Chebyshev Low-Pass Pole-Zero Plot');

figure;

zplane(bC_HP, aC_HP);

title('Chebyshev High-Pass Pole-Zero Plot');

disp('Check pole locations to verify filter stability.');

COMMEND:

CHEBYSHEV TYPE-I LOW-PASS FILTER

Filter Order = 6

Numerator Coefficients:

1.0e-03 *

0.0464    0.2782    0.6956    0.9274    0.6956    0.2782    0.0464

Denominator Coefficients:

1.0000   -4.8694   10.3807  -12.3367    8.5969   -3.3268    0.5586

CHEBYSHEV TYPE-I HIGH-PASS FILTER

Filter Order = 6

Numerator Coefficients:

0.1832   -1.0995    2.7486   -3.6648    2.7486   -1.0995    0.1832

Denominator Coefficients:

1.0000   -2.8266    3.9462   -3.1151    1.6070   -0.5252    0.1384

BUTTERWORTH LOW-PASS FILTER ORDER

12

BUTTERWORTH HIGH-PASS FILTER ORDER

12

Check pole locations to verify filter stability.

OP