Experiment-4

SIMULATION EXPERIMENT – 5

❖ Realization of IIR Filters Using Direct Form-I, Direct Form-II, Cascade and Parallel Structures Using MATLAB

CO Mapping:CO1

Blooms Level K5-Evaluate

WK Level :WK4- Engineering Design (Evaluates alternative filter designs against specifications)

➔ Major Topic

IIR Realization Structures

➔ Minor Topics

1. To realize a specified IIR filter using Direct Form-I and Direct Form-II structures in MATLAB.

➔ Objectives of this Experiment:

1. To realize a given IIR transfer function using Direct Form-I and Direct Form-II structures.

2. To obtain cascade realization using second-order sections.

3. To obtain parallel realization using partial-fraction decomposition.

4. To compare the realized structures using coefficients, impulse/output responses and implementation characteristics.

➔ Expected Outcome from this Experiment:

Students shall be able to:

1. Construct Direct Form-I and Direct Form-II realizations from the same IIR transfer function.

2. Obtain second-order-section coefficients for cascade realization.

3. Obtain parallel branches using partial-fraction decomposition and verify their combined response.

4. Compare the output responses of all realizations and confirm that equivalent structures implement the same transfer function within numerical tolerance.

➔ Software Required:

MATLAB with Signal Processing Toolbox.

➔ Objective linkage:

The experiment is structured so that each objective directly produces a verifiable output listed in the Output Parameter Table.

➔ Experimental Procedure:

1. Select a stable IIR filter and obtain its numerator and denominator coefficient vectors.

2. Implement the transfer function in Direct Form-I and Direct Form-II using MATLAB-compatible difference equations or realization functions.

3. Convert the filter to second-order sections and construct the cascade realization.

4. Perform partial-fraction decomposition and construct the parallel realization.

%% Experiment 5: Realization of IIR Filters (DF-I, DF-II, Cascade, Parallel)

clc; clear; close all;

%% 1. Select a stable IIR filter

Fs = 1000;  fc = 50;  N = 6;

[b, a] = butter(N, fc/(Fs/2));          % H(z) = B(z)/A(z)

disp('--- Transfer function coefficients ---');

disp('b ='); disp(b);

disp('a ='); disp(a);

fprintf('Stable? %d (max |pole| = %.5f)\n', all(abs(roots(a))<1), max(abs(roots(a))));

%% 2. Test inputs

L  = 400;  n = 0:L-1;

x_imp = [1 zeros(1, L-1)];                                  % impulse

x_sig = sin(2*pi*20*n/Fs) + 0.5*sin(2*pi*300*n/Fs);         % 20 Hz + 300 Hz

%% 3. Direct Form-I (separate input and output delay lines)

% y[n] = sum b(k+1) x[n-k] - sum a(k+1) y[n-k]

dfI = @(b,a,x) df1_run(b,a,x);

%% 4. Direct Form-II (one shared delay line w[n])

% w[n] = x[n] - sum a(k+1) w[n-k];  y[n] = sum b(k+1) w[n-k]

dfII = @(b,a,x) df2_run(b,a,x);

yI_imp  = dfI (b, a, x_imp);   yI_sig  = dfI (b, a, x_sig);

yII_imp = dfII(b, a, x_imp);   yII_sig = dfII(b, a, x_sig);

%% 5. Cascade realization (second-order sections)

[sos, g] = tf2sos(b, a);

sos(1,1:3) = sos(1,1:3)*g;              % absorb overall gain into section 1

nSec = size(sos,1);

disp('--- Cascade: SOS matrix [b0 b1 b2 a0 a1 a2] ---'); disp(sos);

casc = @(x) cascade_run(sos, x);

yC_imp = casc(x_imp);   yC_sig = casc(x_sig);

%% 6. Parallel realization (partial fractions in z^-1)

[r, p, k] = residuez(b, a);             % H(z) = k + sum r_i/(1 - p_i z^-1)

tol = 1e-8;

used = false(size(p));

Bp = [];  Ap = [];                      % each row = one real second-order branch

for i = 1:numel(p)

if used(i), continue; end

if abs(imag(p(i))) < tol            % real pole -> first-order branch

Bp = [Bp; real(r(i)) 0 0];

Ap = [Ap; 1 -real(p(i)) 0];

used(i) = true;

else                                % complex-conjugate pair -> 2nd-order branch

j = find(~used & abs(p - conj(p(i))) < 1e-6, 1);

Bp = [Bp; 2*real(r(i))  -2*real(r(i)*conj(p(i)))  0];

Ap = [Ap; 1  -2*real(p(i))  abs(p(i))^2];

used([i j]) = true;

end

end

if isempty(k), k = 0; end

disp('--- Parallel: direct term k ---'); disp(k);

disp('--- Parallel branches ---');

disp('Numerators   [c0 c1 c2]  ='); disp(Bp);

disp('Denominators [1 d1 d2]   ='); disp(Ap);

par = @(x) parallel_run(Bp, Ap, k, x);

yP_imp = par(x_imp);   yP_sig = par(x_sig);

%% 7. Reference output from MATLAB filter()

yR_imp = filter(b, a, x_imp);   yR_sig = filter(b, a, x_sig);

%% 8. Output responses of all realizations

figure;

subplot(2,1,1);

stem(n(1:80), yR_imp(1:80), 'k', 'filled'); hold on;

plot(n(1:80), yI_imp(1:80),  'b-o'); plot(n(1:80), yII_imp(1:80), 'r--s');

plot(n(1:80), yC_imp(1:80),  'g-^'); plot(n(1:80), yP_imp(1:80),  'm-.d');

grid on; legend('filter()','DF-I','DF-II','Cascade','Parallel');

title('Impulse Response of All Realizations'); xlabel('n'); ylabel('h[n]');

subplot(2,1,2);

plot(n, x_sig, 'Color', [0.7 0.7 0.7]); hold on;

plot(n, yI_sig, 'b', n, yII_sig, 'r--', n, yC_sig, 'g', n, yP_sig, 'm-.', 'LineWidth', 1.2);

grid on; xlim([0 200]); legend('Input','DF-I','DF-II','Cascade','Parallel');

title('Output for 20 Hz + 300 Hz Input'); xlabel('n'); ylabel('Amplitude');

%% 9. Frequency responses of the structures

nfft = 2048;

[Hd, f] = freqz(b, a, nfft, Fs);

Hc = ones(nfft,1);

for s = 1:nSec, Hc = Hc .* freqz(sos(s,1:3), sos(s,4:6), nfft, Fs); end

Hp = k * ones(nfft,1);

for s = 1:size(Bp,1), Hp = Hp + freqz(Bp(s,:), Ap(s,:), nfft, Fs); end

figure;

plot(f, 20*log10(abs(Hd)), 'k', 'LineWidth', 2); hold on;

plot(f, 20*log10(abs(Hc)), 'g--', 'LineWidth', 1.5);

plot(f, 20*log10(abs(Hp)), 'm-.', 'LineWidth', 1.5);

grid on; ylim([-120 5]); xline(fc, ':k'); yline(-3, ':k');

legend('Direct form', 'Cascade', 'Parallel');

title('Magnitude Response: Direct vs Cascade vs Parallel');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)');

%% 10. Equivalence check (within numerical tolerance)

err = @(y,yr) max(abs(y - yr));

fprintf('\n--- Max |error| vs filter() (impulse input) ---\n');

fprintf('DF-I     : %.3e\n', err(yI_imp,  yR_imp));

fprintf('DF-II    : %.3e\n', err(yII_imp, yR_imp));

fprintf('Cascade  : %.3e\n', err(yC_imp,  yR_imp));

fprintf('Parallel : %.3e\n', err(yP_imp,  yR_imp));

fprintf('--- Max |error| vs filter() (sinusoid input) ---\n');

fprintf('DF-I     : %.3e\n', err(yI_sig,  yR_sig));

fprintf('DF-II    : %.3e\n', err(yII_sig, yR_sig));

fprintf('Cascade  : %.3e\n', err(yC_sig,  yR_sig));

fprintf('Parallel : %.3e\n', err(yP_sig,  yR_sig));

fprintf('Frequency-response mismatch  Cascade: %.3e   Parallel: %.3e\n', ...

max(abs(Hc-Hd)), max(abs(Hp-Hd)));

tolr = 1e-9;

fprintf('All structures equivalent (tol %.0e)? %d\n', tolr, ...

all([err(yI_imp,yR_imp) err(yII_imp,yR_imp) err(yC_imp,yR_imp) err(yP_imp,yR_imp)] < tolr));

%% 11. Implementation characteristics

M = numel(b)-1;  D = numel(a)-1;

mult_DF  = (M+1) + D;      add_DF = M + D;                 % DF-I and DF-II

mult_C   = 5*nSec;         add_C  = 4*nSec;                % 5 mult, 4 add per biquad

nB = size(Bp,1);

mult_P   = 3*nB + 2*nB + 1;  add_P = 3*nB;                 % rough count, 2nd-order branches

fprintf('\n=========== IMPLEMENTATION COMPARISON ===========\n');

fprintf('%-12s %12s %12s %12s\n', 'Structure', 'Delays', 'Multipliers', 'Adders');

fprintf('%-12s %12d %12d %12d\n', 'Direct I',  M + D, mult_DF, add_DF);

fprintf('%-12s %12d %12d %12d\n', 'Direct II', max(M,D), mult_DF, add_DF);

fprintf('%-12s %12d %12d %12d\n', 'Cascade',   2*nSec, mult_C, add_C);

fprintf('%-12s %12d %12d %12d\n', 'Parallel',  2*nB, mult_P, add_P);

%% 12. Coefficient quantization (4 significant digits) - sensitivity

q = @(x,d) sign(x).*round(abs(x).*10.^(d-1-floor(log10(abs(x)+(x==0))))) ...

.*10.^-(d-1-floor(log10(abs(x)+(x==0))));

bq = q(b,4);  aq = q(a,4);

Hq_d = freqz(bq, aq, nfft, Fs);

Hq_c = ones(nfft,1);

for s = 1:nSec

Hq_c = Hq_c .* freqz(q(sos(s,1:3),4), q(sos(s,4:6),4), nfft, Fs);

end

figure;

plot(f, 20*log10(abs(Hd)),   'k',   'LineWidth', 2); hold on;

plot(f, 20*log10(abs(Hq_d)), 'r--', 'LineWidth', 1.5);

plot(f, 20*log10(abs(Hq_c)), 'g-.', 'LineWidth', 1.5);

grid on; ylim([-150 5]);

legend('Ideal', 'Direct form (quantized)', 'Cascade (quantized)');

title('Effect of Coefficient Quantization (4 significant digits)');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)');

fprintf('\n--- Quantized structures ---\n');

fprintf('Direct form: max |pole| = %.5f, stable = %d, max response error = %.3e\n', ...

max(abs(roots(aq))), all(abs(roots(aq))<1), max(abs(Hq_d-Hd)));

fprintf('Cascade    : max response error = %.3e\n', max(abs(Hq_c-Hd)));

%% 13. Pole-zero plot

figure; zplane(b, a); grid on; title('Pole-Zero Plot of H(z)');

%% ------------------------ Local functions ------------------------

function y = df1_run(b, a, x)

b = b(:).'/a(1);  a = a(:).'/a(1);

M = numel(b)-1;   D = numel(a)-1;

y = zeros(size(x));

for n = 1:numel(x)

acc = 0;

for kk = 0:M

if n-kk >= 1, acc = acc + b(kk+1)*x(n-kk); end

end

for kk = 1:D

if n-kk >= 1, acc = acc - a(kk+1)*y(n-kk); end

end

y(n) = acc;

end

end

function y = df2_run(b, a, x)

b = b(:).'/a(1);  a = a(:).'/a(1);

K = max(numel(b), numel(a)) - 1;

b = [b zeros(1, K+1-numel(b))];  a = [a zeros(1, K+1-numel(a))];

w = zeros(1, K+1);                  % w(1)=w[n], w(2)=w[n-1], ...

y = zeros(size(x));

for n = 1:numel(x)

w(1) = x(n) - sum(a(2:end).*w(2:end));

y(n) = sum(b.*w);

w(2:end) = w(1:end-1);          % shift delay line

end

end

function y = cascade_run(sos, x)

y = x;

for s = 1:size(sos,1)

y = df2_run(sos(s,1:3), sos(s,4:6), y);

end

end

function y = parallel_run(Bp, Ap, k, x)

y = k*x;

for s = 1:size(Bp,1)

y = y + df2_run(Bp(s,:), Ap(s,:), x);

end

end