Experiment-4

SIMULATION EXPERIMENT – 4

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters Using MATLAB

CO Mapping:CO1

Blooms Level K5-Evaluate

WK Level :WK4- Engineering Design (Evaluates alternative filter designs against specifications)

➔ Major Topic

Elliptic IIR Design

➔ Minor Topics

1. To design Butterworth, Chebyshev Type-I and Elliptic IIR filters for identical frequency specifications.

➔ Objectives of this Experiment:

1. To design Butterworth, Chebyshev Type-I and Elliptic IIR filters for comparable specifications.

2. To examine passband/stopband ripple, filter order and transition bandwidth for the three approximations.

3. To obtain and compare magnitude responses and filter orders.

4. To evaluate the trade-off between ripple, transition sharpness and filter complexity.

➔ Software Required:

MATLAB with Signal Processing Toolbox.

➔ FORMULA TO BE USED:

Butterworth:  |H(jΩ)|² = 1 / [1 + (Ω/Ωc)²ᴺ].

Chebyshev-I:  |H(jΩ)|² = 1 / [1 + ε²Tₙ²(Ω/Ωp)].

Transition bandwidth:  ΔF = Fstop - Fpass.

Percentage deviation:  % deviation = |Measured - Specified| / |Specified| × 100%.

➔ Experimental Procedure:

1. Define a common sampling frequency, passband edge, stopband edge, passband ripple and stopband attenuation.

2. Determine the minimum order and cutoff parameters for each approximation.

3. Design Butterworth, Chebyshev Type-I and Elliptic filters.

4. Generate magnitude and phase responses and display the three magnitude responses on a common plot.

5. Record filter order, transition width, passband/stopband ripple and edge frequencies.

6. Prepare a comparative table and explain the observed design trade-offs.