1.Design, Realization and Performance Analysis of Butterworth IIR Low-Pass and High-Pass Digital Filters Using MATLAB

CODE ;

clc;

clear;

close all;

% Input Specifications

Fs = 1000; % Sampling frequency (Hz)

Fp_LP = 100; % Low-pass passband edge (Hz)

Fst_LP = 150; % Low-pass stopband edge (Hz)

Fst_HP = 200; % High-pass stopband edge (Hz)

…figure;

zplane(b_LP, a_LP);

title('Low-Pass Filter Pole-Zero Plot');

figure;

zplane(b_HP, a_HP);

title('High-Pass Filter Pole-Zero Plot');

disp('Low-Pass Filter is stable if all poles lie inside the unit circle.');

disp('High-Pass Filter is stable if all poles lie inside the unit circle.');

COMMEND:

LOW-PASS BUTTERWORTH FILTER

Filter Order = 12

Numerator Coefficients:

1.0e-03 *

0.0002    0.0027    0.0146    0.0488    0.1098    0.1757    0.2050    0.1757    0.1098    0.0488    0.0146    0.0027    0.0002

Denominator Coefficients:

1.0000   -6.8844   22.4341  -45.5100   63.7725  -64.8501   48.9643  -27.6092   11.5218   -3.4662    0.7128   -0.0899    0.0053

HIGH-PASS BUTTERWORTH FILTER

Filter Order = 17

Numerator Coefficients:

Columns 1 through 16

0.0001   -0.0013    0.0100   -0.0502    0.1756   -0.4566    0.9132   -1.4350    1.7938   -1.7938    1.4350   -0.9132    0.4566   -0.1756    0.0502   -0.0100

Columns 17 through 18

0.0013   -0.0001

Denominator Coefficients:

Columns 1 through 16

1.0000   -0.5247    2.4322   -1.0556    2.2461   -0.7954    1.0097   -0.2858    0.2357   -0.0517    0.0283   -0.0046    0.0016   -0.0002    0.0000   -0.0000

Columns 17 through 18

0.0000   -0.0000

Low-Pass Filter is stable if all poles lie inside the unit circle.

High-Pass Filter is stable if all poles lie inside the unit circle.

OP: